REPUBLIC OF YEMEN

SANA'A UNIVERSITY

FACULTY OF ENGINEERING

MECHATRONICS ENGINEERING DEPARTMENT

DC Motor Speed Control Using PID

Done by:

Musa   ALsalmani 202174202

Hasan Moqbel   202274198

Hamzah Alkhalaf   202274127

System (Parallel)

Supervised by:

Eng. Said Al-Samai

1. Introduction	2

2. System Identification (Z-Domain)	3

a. Modeling the Plant (DC Motor)	3

b. Discretization	3

c. Experimental Data Collection	3

3. Open-Loop (O.L.) Analysis	3

a. Step Response Analysis	3

b. Frequency Response Analysis	4

4. Controller Design (Lead-Lag Compensators)	4

a. Lead Compensator	4

b. Lag Compensator	4

c. Design Approach	5

5. PID Tuning Methods	5

a. Trial and Error	5

b. PID Tuner (MATLAB or Python)	5

c. Ziegler–Nichols Method	5

6. Closed-Loop (C.L.) Analysis	5

a. Transient Analysis	5

b. Frequency Response	6

c. Steady-State Error	6

7. Implementation (AVR)	6

a. ATmega328P Implementation	6

b. Proteus Simulation	6

c. Hardware Implementation	6

8. Conclusion	6

1. Introduction

DC motors are widely used in automation and control applications, and precise speed control is often required to achieve optimal performance in various systems. One of the most effective approaches for controlling the speed of a DC motor is through a Proportional-Integral-Derivative (PID) controller, which adjusts the control input based on feedback to reach the desired output speed. This report covers the steps involved in identifying the system, designing and tuning the PID controller, and implementing it on a microcontroller.

2. System Identification (Z-Domain)

To design an effective PID controller, we must first identify the transfer function of the DC motor system in the Z-domain. The following steps outline the process:

a. Modeling the Plant (DC Motor)

A DC motor can be represented by a first-order system, with the transfer function G(s) in the Laplace domain defined as:

where:

- K is the gain of the system.

- τ (tau) is the time constant of the system, which represents the motor's response time.

b. Discretization

To implement a digital controller, the continuous-time transfer function G(s)G(s)G(s) is converted to the discrete-time domain G(z)G(z)G(z) using techniques such as Zero-Order Hold (ZOH) or Bilinear Transformation. This enables the use of digital controllers, typically implemented on microcontrollers.

c. Experimental Data Collection

- A step input is applied to the motor using Pulse Width Modulation (PWM).

- The motor's response is recorded using a rotary encoder to measure the output speed.

- System identification techniques are used to estimate the parameters KKK and τ\tauτ, which are essential for defining the transfer function.

3. Open-Loop (O.L.) Analysis

a. Step Response Analysis

The open-loop step response provides insights into the motor's performance characteristics, such as:

- Settling time: The time taken for the response to settle within a specific range.

- Rise time: The time taken to reach a certain percentage of the final value.

- Steady-state error: The final error between the desired and actual output.

b. Frequency Response Analysis

Using Bode and Nyquist plots, we analyze the system’s frequency response to gain insights into:

• 	Gain margin and phase margin: These metrics are critical for assessing the system's stability in the open-loop configuration.

4. Controller Design (Lead-Lag Compensators)

Lead-lag compensators are often used alongside PID controllers to refine system behavior.

a. Lead Compensator

The lead compensator improves the phase margin and transient response of the system. Its transfer function is given by:

where τD\tau_{D}τD and α\alphaα are design parameters.

b. Lag Compensator

The lag compensator enhances steady-state accuracy by increasing the system gain. The transfer function for a lag compensator is:

where τI\tau_{I}τI and

β\betaβ are parameters chosen to improve steady-state error.

c. Design Approach

The root locus or frequency domain methods can be used to design the compensators to meet specified transient and steady-state performance criteria.

5. PID Tuning Methods

After identifying the plant and designing initial compensators, we proceed with PID tuning to refine the controller.

a. Trial and Error

The controller parameters Kp, Ki, and Kd are adjusted manually to achieve the desired system performance. This approach requires repeated testing and observation.

b. PID Tuner (MATLAB or Python)

Tools like MATLAB or Python's control libraries can be used to automate the tuning process. These tools help optimize the parameters based on performance criteria like settling time and overshoot.

c. Ziegler–Nichols Method

The Ziegler–Nichol’s method provides an empirical approach to tuning. The steps include:

- Identifying the ultimate gain Ku and ultimate period Tu by increasing K until sustained oscillations are observed.

- Using Ku and T to calculate Kp, Ki, and Kd values based on Ziegler–Nichols formulas for a PID controller.

6. Closed-Loop (C.L.) Analysis

a. Transient Analysis

With the PID controller in place, the system’s transient performance is analyzed:

• 	Settling time • 	Overshoot • 	Rise time

b. Frequency Response

We assess the closed-loop system's gain and phase margins to verify stability and robustness.

c. Steady-State Error

To ensure accurate speed tracking, the system’s steady-state error is evaluated for various setpoints.

7. Implementation (AVR)

To implement the PID controller, we selected the ATmega328P microcontroller, using the following steps:

a. ATmega328P Implementation

The PID control algorithm is developed in C and executed in real-time using timer interrupts to maintain a fixed sampling period.

b. Proteus Simulation

The entire system, including the motor, sensors, and controller, is simulated in Proteus to validate the design and ensure proper communication and control performance.

c. Hardware Implementation

The control system is constructed using the ATmega328P, ESP32, L298N motor driver, and rotary encoder. Testing is conducted on the actual hardware, with parameters adjusted as necessary to ensure effective speed control.

8. Conclusion

A PID controller offers effective speed control for a DC motor, and tuning methods like Ziegler–Nichols and trial-and-error enable optimized performance. Lead and lag compensators further enhance transient and steady-state performance. Through simulation and hardware testing, we achieve robust and stable motor speed control, meeting both performance and implementation requirements.